WARPED PINBALL  ·  SETUP GUIDE

Get Trench Coat

Trench Coat reprograms the operating system and application software on any Vector board. Download the app for your computer, then follow the steps below to get started.

# Step 1  —  Download the file for your computer

Pick the one that matches your machine and click its link. That's the whole app — one file, nothing to install.

| Your computer | Who it's for | Download |
| --- | --- | --- |
| Windows | Any Windows 10 or 11 PC. | Download ↓ TrenchCoat-windows.exe |
| Mac – Apple Silicon | Newer Macs (2020 and later) with an M1, M2, M3 or M4 chip. | Download ↓ TrenchCoat-macos-arm64 |
| Mac – Intel | Older Macs (before 2020) with an Intel processor. | Download ↓ TrenchCoat-macos-x86_64 |
| Linux | Most 64-bit desktop Linux distributions. | Download ↓ TrenchCoat-linux |

Not sure which Mac?  Click the Apple menu (top-left) → About This Mac. If it lists a chip named Apple M1/M2/M3/M4, choose Apple Silicon. If it says Intel, choose Intel.

# Step 2  —  Open it the first time

Your computer will warn you the first time you run it, because this app isn't registered with Microsoft or Apple. It's safe — you just need to tell your system to allow it. You only do this once.

Windows

Double-click TrenchCoat-windows.exe in your Downloads folder.

A blue box says “Windows protected your PC.” Click the small More info link.

A new button appears — click Run anyway.

Heads up:  A black command window opens when it runs — that's normal, that's the app.

Mac  (both Apple Silicon & Intel)

Find the downloaded file in Finder (usually your Downloads folder).

Right-click the file (or hold Control and click) and choose Open.

A warning appears — this time it includes an Open button. Click it.

If it's blocked:  On the newest macOS, double-click once, then go to Apple menu → System Settings → Privacy & Security, scroll down, and click Open Anyway. It runs in the Terminal app — that's expected.

Linux

Open a terminal in the folder where you downloaded the file.

Make it runnable:  chmod +x TrenchCoat-linux

Run it:  ./TrenchCoat-linux

# Step 3  —  Plug in your Vector board

With the app open, connect the Vector board so it can be programmed. This part is easy — but read the warning first.

⚠  LEAVE THE PINBALL MACHINE OFF

Do NOT turn the pinball machine on. It must stay powered off the entire time you use Trench Coat.

Leave the pinball machine turned OFF. You do NOT need to remove the Vector board from the machine — just leave it where it is.

Using a USB cable, plug the Vector board directly into your computer (PC, Mac, or Linux).

The app will detect the board. Follow the on-screen prompts to select your board type.

Reminder:  The Vector board is powered through the USB cable from your computer, so the pinball machine never needs to be switched on.

Trouble downloading?  All files also live on the official releases page.

Trench Coat is open-source and not code-signed, which is why your computer shows a first-run warning. The download links above always give you the latest version.